Supplementary Note 1

The SRM used for annotation and identification had a concentration of 0.47 ng/mL per chemical. Thus, we used the spectral intensities associated with these known concentrations to estimate chemical concentrations in tissues in parts per million (ppm) and parts per billion (ppb). It should be noted that the original data processing pipeline used a nominal concentration of 0.5 ng/mL for calibration calculations; therefore, a correction factor of 0.94 (0.47/0.5) was applied to all concentration estimates prior to ppm and ppb calculations to account for the true SRM concentration. Importantly, we relied on the following experimental features and assumptions:

- Tissues are processed via solvent extraction and subsequent solvent evaporation, followed by reconstitution in 50 μL of isooctane. For the purposes of calculating concentrations in the original samples, we assume 100% extraction efficiency from tissues and plasma, such that the entire amount of the environmental chemical present in the sample is recovered in the isooctane.

- A chemical concentration of 0 ng/mL results in an intensity of 0, representing the origin (0,0). This is used in combination with the intensity from the known concentration in the SRM (0.47 ng/mL) to establish a two-point standard curve, which assumes linearity.

- While each chemical has a concentration in the SRM of 0.47 ng/mL, the concentration in 50 μL of isooctane is 1.88 ng/mL. This is because 200 μL of SRM plasma is used, and therefore, 0.094 ng of the total environmental chemical of interest is extracted. This is dried after extraction and reconstituted in 50 μL of isooctane, resulting in a concentration of 1.88 ng/mL in the injected solution.

Our calculations to estimate tissue concentrations were performed as follows for each individual chemical:

- A linear model is fit based on the assumptions discussed above, such that:

y=βx

- where y is the known concentration of the environmental chemical in the SRM (0.47 ng/mL for all environmental chemicals analyzed), x is the observed spectral intensity for the given environmental chemical, and β is the slope of the calibration curve, defined by the line through the origin (0,0) and the point x,y.

- Once the slope (β) for the specific chemical is established, it is then used to estimate concentration (ng/mL) in the sample (Ce) using the following equation:

Ce=βI

- where I is the observed spectral intensity for the given environmental chemical in the sample. However, to determine the true concentration in the isooctane solvent (Ces), we must scale Ce by a factor of 4 to account for the concentration that occurs during extraction. Specifically, the 200 μL of plasma standard extracted contains 0.094 ng total of the environmental chemical (0.47 ng/mL). Assuming 100% extraction, this 0.094 ng was then resuspended in 50 μL isooctane (1.88 ng/mL), Therefore, we use a 4× scaling factor:

Ces=4Ce

- Next, the Ces is converted from ng/mL to mg/L:

Ces (mg/L)=Ces (ng/mL)×103 mLL×10-6 mgng

Ces (mg/L)=Ces (ng/mL)×10-3

- Next, the total mass of the chemical of interest present in 50 μL of isooctane (MEC) is determined:

MEC (mg)=Ces (mg/L)×50 μL solvent×10-6 LμL

MEC (mg)=5×10-5 (L)×Ces (mg/L)

- To derive the mass fraction of the environmental chemical in tissue, we divide MEC (mg) by MT (mg) and scale to ppm or ppb:

ppm=MECMT×106

ppb=MECMT×109

Alternatively, when algorithm outputs are used directly without unit conversions, the following formula can be used, provided Ce is in ng/mL and MT is in mg:

ppm=Ce×102MT

ppb=Ce×105MT